IT 125-41194

Alex Arnold

4-21-25

Lab 13.3.12

Part 1: Troubleshoot the Network

Step 1: Test connectivity.

Step 2: Investigate issues and record findings.

| Device | Network Home/Enterprise | Issue | Remedy |
| --- | --- | --- | --- |
| Laptop | Home | Cant connect to web server | blank |
| Smart Phone | Home | Cant connect to web server |  |
| Tablet PC | Home | Cant ping Router |  |

Laptop

Laptop cant connect to webserver but can ping router

Smartphone is the same as the laptop

The tablet can not connect to either the router or the web server

Tablet pc was set to static IP –

Static changed to DHCP

Tablet is now at the same state as the other devices on the home network

Upon investigation internet type is set to static IP when it should be Automatic DHCP and the static DNS 1 was incorrect it should have ended in .254.

Laptop can now ping webserver

Smartphone can now ping webserver

Tablet can now ping webserver

All devices on the home network can now connect to the web server

Enterprise network

Laptop 2 can neither ping the web server or the WLC-1

Same goes for Laptop 1

Laptop 1 not connected to wireless but the configuration is correct and WLAN10 is showing as a network

Laptop 2 is not connected to wireless and the WLAN20 network does not show up

Vlan 20 was disabled

enabled and saved

Laptop 2 profile was misconfigured as user 20 instead of user2

Vlan 10 was set to WPA2 802.1x when it should be WPA PSK

Fixed

Laptop 1 is now connected

Laptop 1 can now ping the web server

Laptop 1 can connect to the web server as well

All configs are correct but laptop 2 can connect to the network